DIGITAL DESIGN

ASSIGNMENTREPORT

ASSIGNMENT ID : 04

Student Name: Weibao Fu

Student ID: 11812202

## PART 1: DIGITAL design THEORY

1.

JK filp-flop

| J | K | Q | Qt+1 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 |

Characteristic equation: Qt+1 = JQ’ + K’Q

T flip-flop

| T | Q | Qt+1 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

Characteristic equation: Qt+1 = TQ’ + T’Q

2.

Circuit diagram

State table

| x | y | A | B | JA | KA | JB | KB | z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |

State equation

From the state table, we can get At+1 = JAAt’ + KA’At

= (Btx + Bt’y’)At’ + (Bt’xy’)’At

= At’Bt’y’ + Btx + Atx’ + Aty

Bt+1 = JBBt’ + KB’Bt

= (At’x)Bt’ + (At + xy’)’Bt

= At’Bt’x + At’Bt(x’+y)

3.

D

Since the state equation of JK flip-flop is Q = JQ’ + K’Q

Then we can use multiplexer, we can set Q0 = J, Q1 = K to implement

Y = JQ0’ + K’Q1 , and we use a D flip-flop to finish the JK flip-flop.

4.

State table

| At | Bt | TA | TB | At+1 | Bt+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 |

State equation

TA = A + B

TB = A’ + B

At+1 = TAA’ + TA’A

= (A + B)A’ + (A+B)’A

= A’B

Bt+1 = TBB’ + TB’B

= (A’ + B)B’ + (A’ + B)’B

= A’B’

11

State transfer graph

From the state transfer graph and state table, we can get that the circuit is a ternary counter. It will loop in every 3 periods.

## PART 2: DIGITAL design LAB (Task1)

Design

Verilog Code

`timescale 1ns / 1ps
module T_flipflop(T,Clk,rst,Q,Qn);
input  T,Clk,rst;
output Q,Qn;
wire q1,qn,temp1,temp2,temp3,temp4;
nand G7(temp1,~Clk,T,Qn,rst);
nand G8(temp2,~Clk,T,Q);
nand G5(q1,temp1,qn);
nand G6(qn,temp2,q1,rst);
nand G3(temp3,q1,Clk);
nand G4(temp4,qn,Clk);
nand G1(Q,temp3,Qn);
nand G2(Qn,temp4,Q,rst);
endmodule

simulation

Verilog Code (Simulation)

`timescale 1ns / 1ps
module T_sim( );
reg Clk,T,rst;
wire Q,Qn;
T_flipflop usim(T,Clk,rst,Q,Qn);
initial
begin
   {rst,Clk,T} = 3'b010;
   #10 rst = 1'b1;
   repeat(16)
   #10 {Clk,T} = {Clk,T}+1;
   $finish;
end
endmodule

Wave Form (rst is low level effective)

When 0-10ns, rst = 0, then Q=0, Qn=1

When 10-170ns, rst =1:

When 50ns, T is 1, and Clk is in posedge, Qt = 0, so Qt+1 = 1 and Qt = 0

When 90ns, T is 1, and Clk is in posedge, Qt = 1, so Qt+1 = 0 and Qt = 1

When 130ns, T is 1, and Clk is in posedge, Qt = 0, so Qt+1 = 1 and Qt = 0

From the wave form, we can get when Clk is in posedge and T=1, the state of Q will change, else the state of Q will not change. The result is satisfied T filp-flop.

Constraint file and the testing

Constraint File

Analysis

Testcase #1

When rst is 0,

Then Q = 0, Qn = 1

Q  Qn

rst   Clk   T

Testcase #2

Although Clk and T are 1,

Q = 0, Qn = 1 since rst = 0

Q  Qn

rst   Clk   T

Testcase #3

When rst is 1, Clk and T are 1

Then the state will change,

Q = 1, Qn = 0

Q  Qn

rst   Clk   T

Testcase #4

When rst is 1, Clk and T are 1

Then the state will change,

Q = 0, Qn = 1

Q  Qn

rst   Clk   T

Testcase #5

When rst is 1, Clk is 1,

Then the state will not change since T=0, Q = 0, Qn = 1

Q  Qn

rst   Clk   T

Testcase #6

When rst is 1, Clk and T are 1

Then the state will change,

Q = 1, Qn = 0

Q  Qn

rst   Clk   T

Testcase #7

Although Clk and T are 1,

the state will reach original state since rst = 0, Q = 0, Qn = 1

Q  Qn

rst   Clk   T

## PART 2: DIGITAL design LAB (Task2)

Design

Verilog Code

`timescale 1ns / 1ps
module task2(Clk,rst,x_in,y_out);
input Clk,rst;
input [4:0]x_in;
output reg[2:0]y_out;
reg [2:0]y_next;
reg sum=0;
parameter A=3'b001,B=3'b010,C=3'b100;
always @(posedge Clk,negedge rst) begin
   if(~rst) y_next <= A;
   else begin
      sum = sum^x_in[0]^x_in[1]^x_in[2]^x_in[3]^x_in[4];
      if(sum)  y_next <= C;
      else y_next <= B;
   end
end
always @(y_next) begin
   y_out <= y_next;
end
endmodule

simulation

Verilog Code (Simulation)

`timescale 1ns / 1ps
module task2_sim( );
reg Clk,rst;
reg [4:0]x_in;
wire [2:0]y_out;
task2 usim(Clk,rst,x_in,y_out);
initial begin   
   {Clk,rst,x_in}=7'b0000000;
   while({rst,x_in}<6'b111111)
   #10 {rst,x_in} = {rst,x_in}+1;
   #10 $finish;
end
always #10 Clk=~Clk;
endmodule

Wave Form (rst is low level effective)

From the wave form:

When 0-320ns, rst = 0, so y_out is state A(001)

When 320-640ns, rst=1:

When 330ns, the clock is in posedge, x_in is (00001), the sum of 1 is 1, so the state is C(100)

When 350ns, the clock is in posedge, x_in is (00011), the sum of 1 is 3, so the state is C(100)

When 370ns, the clock is in posedge, x_in is (00101), the sum of 1 is 5, so the state is C(100)

When 390ns, the clock is in posedge, x_in is (00111), the sum of 1 is 8, so the state is B(010)

When 410ns, the clock is in posedge, x_in is (01001), the sum of 1 is 10, so the state is B(010)

When 430ns, the clock is in posedge, x_in is (01011), the sum of 1 is 13, so the state is C(100)

……(Others are ignored)

From the wave form, we can conclude that if rst is effective, then the output state is A. Else we read the information of in_x, and count the total number of 1, when the count is odd, the output state is C, else the output state is B. Such that, the result is satisfied.

Constraint file and the testing

Constraint File

Analysis

(In order to make the result looks more comfortable, I swap the order of (A,B,C) as below. Which means I link the higher level port with “M13”, lower with “K17”)

Clk   rst

Testcase #1:

When rst=0, the sum of 1 is 0

The state is A (001)

A   B  C

Clk   rst

Testcase #2:

When rst=1, and Clk is in posedge,  x_in(10000), the sum of 1 is 1

The state is C (100)

A  B  C

Clk   rst

Testcase #3:

When rst=1, and Clk is in posedge,  x_in(11000), the sum of 1 is 3

The state is C (100)

A  B  C

Clk   rst

Testcase #4:

When rst=1, and Clk is in posedge,  x_in(11100), the sum of 1 is 6

The state is B (010)

A  B  C

Clk   rst

Testcase #5:

When rst=1, and Clk is in posedge,  x_in(11111), the sum of 1 is 11

The state is C (100)

A  B  C

Clk   rst

Testcase #6:

When rst=0, and Clk is in posedge,  Although x_in(11111), the sum of 1 is still 11 since rst = 0

The state is A (001)

A  B  C

Clk   rst

Testcase #7:

When rst=1, and Clk is in posedge,  x_in(11111), the sum of 1 is 16

The state is B (010)

A  B  C

the description of operation

No problem